TAP 106- 3: Kinds of light bulb

People use electric light bulbs for many purposes, from a torch used to light up a path home, to aircraft searchlights. These lamps differ tremendously in the power they use.

1.	All bulbs are stamped with two different values, for instance 36 W, 12 V. What do these numbers tell you?

2.	You can also use these values to calculate the current through the bulb filament. The table below shows these values for five different bulbs. Use a suitable formula to calculate the missing values.

| Bulb | Power / W | pd / V | Current / A |
| --- | --- | --- | --- |
| Headlamp | 36 | 12 |  |
| Torch bulb | 0.09 | 3 | 0.03 |
| Filament bulb | 100 | 230 |  |
| Flashlight bulb | 4.5 | 9 | 0.5 |
| Energy Saving bulb | 24 | 230 |  |

Fuse protection

3.	Explain why appliances are protected by a fuse and explain how the fuse provides this protection.

4.	The table shows the power rating and voltage as marked on a number of appliances. Calculate the operating current of each appliance. Suggest a suitable fuse value for each appliance choosing from the fuse values given.

| Appliances | Power rating | pd / V | Operating current / A | Suggested fuse values choosing from 3 A; 13 A |
| --- | --- | --- | --- | --- |
| Iron | 1200 W | 230 |  |  |
| Vacuum cleaner | 900 W | 230 |  |  |
| Headlamp | 48 W | 12 |  |  |
| Jug kettle | 2.4 kW | 230 |  |  |
| Radio | 100 W | 230 |  |  |
| Travel kettle | 340 W | 120 |  |  |
| Microwave cooker | 1.4 kW | 230 |  |  |

Answers and worked solutions

1.	Power in watts – rate of energy use; operating potential difference in volts, pd at which it is designed to work.

2.

| Bulb | Power / W | pd / V | Current / A |
| --- | --- | --- | --- |
| Headlamp | 36 | 12 | 3.0 |
| Torch bulb | 0.09 | 3 | 0.03 |
| Filament bulb | 100 | 230 | 0.43 |
| Flashlight bulb | 4.5 | 9 | 0.5 |
| Energy Saving bulb | 24 | 230 | 0.10 |

3.	To prevent too much power passing through the cable, the fuse is to protect the cable not the appliance (the fuse melts).

4.

| Appliances | power rating | pd / V | operating current / A | Suggested fuse values choosing from 3 A; 13 A |
| --- | --- | --- | --- | --- |
| Iron | 1200 W | 230 | 5.2 | 13 |
| Vacuum cleaner | 900 W | 230 | 3.9 | 13 |
| Head lamp | 48 W | 12 | 4.0 | 13 |
| Jug kettle | 2.4 kW | 230 | 10 | 13 |
| Radio | 100 W | 230 | 0.43 | 3 |
| Travel kettle | 340 W | 120 | 2.8 | 3 |
| Microwave cooker | 1.4 kW | 230 | 6.1 | 13 |

External references

This activity is taken from Advancing Physics Chapter 2, 30S